Magellan Meridian 
PRINT & TAKE THIS WITH YOU ON THE ROUTE

A. To turn On/Off:

1. For power, press the red  “PWR” button 
2. Press the (   “enter” button on the top right to continue
3. Wait for satellite link to give position
4. If at any time you hit the wrong key you can press the X button to back up
B. To Activate a route for navigation:
1. Press the  “MENU”  button

2. Scroll down and highlight “Routes”

3. Press “ENTER”

4. Scroll to select the route number you wish to activate

5. Press the “MENU” 

6. Highlight  “ACTIVATE ROUTE”   and press enter

7. Route is now active

8. To change routes during the course, repeat steps 1-6

C. To Navigate a route

1. Press  “NAV”  button  until you reach the compass page with an arrow 

2. Route is active and GPS should be going to the next WPT

3. READ the next WPT description so you know what to look for when you get there

**** HELPFUL HINT: CHANT WPT NUMBER IN YOUR HEAD SO YOU NEVER           

FORGET  WHICH ONE YOU ARE GOING TO!

4. Follow the arrow and watch the distance count down as you approach WPT

5. Check the mileage and follow directions on description chart

6. Repeat these steps for every WPT

7. If GPS switches to a waypoint other than one you are going to, read and follow instructions in next (GOTO) section

D. GO TO a WPT:

If the GPS switches to a different WPT than you are chanting, you need to GO TO the waypoint you are chanting to resume the course.  Follow these steps to GO TO a WPT:

1. Press the “GOTO” button

2. Use up/down arrow to highlight “user”

3. Press “ENTER”

4. Scroll and highlight the WPT you wish to go to

5. Press  “ENTER”

6. Follow arrow to WPT, then the GPS will automatically resume course

Abbreviations:
ST – Single Track (NOT straight)                                 T-L – Left at the T-intersection 

R – Right                                                                       Y-R – Right at the Y intersection 

L – Left                                                                          Cont. – abbreviation for “Continue”

SR – Stay Right                                                             Rd. – road

SL – Stay Left                                                               Directional –Points you in the right direction 

N, S, E, W – North, South, East, West                          WPT – waypoint 

NE or NW – Northeast or Northwest                              RTE – route 

SE or SW – Southeast or Southwest

Course Description Charts:
· X marks the spot where it's easy to miss a waypoint

· Cautions are in the course description charts; not marked on route; watch for them

· The charts will tell you where the arrow may be misleading or point backwards momentarily
· Mileage will never be the same from bike to bike so update your mileage when you are positive you are ON a waypoint
